527 CMR 13.00 Uniform Blasting Site Detail Check List

Location: ______________________________________________	Date: ___/___/____

Blaster’s Name: ________________________________________	Cert. #: _________

Company Name: _______________________________________	Time of Blast: __:__

Check List					Ref. #			Violations?	YES	NO

Two Way Radio/Warning Signs		CMR 13.09(1)(p)

(“Blasting Zone” “Turn off 2-way Radio”)

Transport Vehicle(s)				CMR 13.06(2), 13.04(3)

(Placards, Fire Marshal Magazine Permit, Attended)

Site Storage (Day Box)			CMR 13.04(3)

(Fire Marshal Magazine Permit, Attended)

NO smoking or open flames			CMR 13.09(1)(d) 2., 3.

(within 50ft of explosives)

NO unnecessary personnel on		CMR 13.09(2)(a)

the blast site (while boreholes are being loaded or are loaded with explosives)

Prior to blasting, excess explosives		CMR 13.09(2)(f)

returned to proper storage

Seismograph must be placed between	CMR 13.09 (9)(f)

5&10 ft of nearest inhabited structure

Explosives, persons & equipment must	CMR 13.09(3)(a), (h)

be at a safe distance prior to blast

Warning signal (3 long blasts 5 min before blast)	CMR 13.09(1)(m), (3)(h)(2)

Blast Signal (2 blasts 1 min before blast)

All Clear Signal (1 prolonged blast)

Post Blast Inspection				CMR 13.09(4)

(blaster must inspect site prior to personnel returning)

Trash (boxes, bags, non-electric)		CMR 13.09(6)

(shall be picked up and/or destroyed)

Seismograph Readings:

PPV:	H_____	V_____	R_____  (2.0 in/sec max)

HZ:	H_____	V_____	R_____ 	Airblast: ______Db (133max)

Report any incident involving flyrock, whether or not was an injury or damage, to the Office of the State Fire Marshal at 978-567-3375.

FP-55 (Rev. May ’10)